H4500 Measuring for Mounting Types

Surface:

Cassette Width: Take 3 measurements: top, center, bottom

To the widest add: 4 1/2” total (width of the track is 2").

Track Height: Full opening height, round up to next foot

Between Jamb:

Cassette Width: Take 3 measurements: top, center, bottom

From the smallest deduct: 3/8” on each side, so 3/4" in total.

That is so you have some adjustability in track placement.

Angle brackets: 2” x 1”, provide a surface to mount the tracks to. Order

them to span the full opening height, rounding up to nearest foot. Then cut

to size at site.

Track height: Cassette is 4.5” high, so order track to height of opening and you

have 4.5” of error margin.

Recessed:

Cassette width:

a) Measure top, middle and bottom

b) Take widest measurement, add 2 1/16” on each side, so total 4 1/8” added

Cassette box: If cassette is recessed in a box, it should minimum be 6” wide and

9 1/2” height for access, and slightly longer than the cassette/roller tube you are mounting in the pocket.

Recessed Track Size: 2" x 1.25"

Track recesses: Should be 2 1/16” deep and 1 ¼” + 1/16”  wide to accommodate

the recessed track

Track height: At least height of opening, possibly more depending on

placement of cassette if it is in a box above opening